Electrons move from high → low concentration

Given:

n(x)=NceEF-Ec(x)kT

Since equilibrium ⇒ no net current:

Jn=0

Total electron current:

Jn=qnμnE+qDndndx

nμnE+Dndndx=0

Electric field is related to conduction band:

E=-1qdEcdx

Substitute:

nμn 1qdEcdx+Dndndx=0
-μnnqdEcdx+Dndndx=0

Differentiate n(x)

n=NceEF-EckT

Take derivative:

dndx=n01kTdEcdx

Substitute into current equation

-μnnqdEcdx+Dnn 1kTdEcdx=0

Factor:

-ndEcdxμnqDnkT=0

For non-trivial solution:

Dnμn=kTq

Similarly for holes:

Dpμp=kTq